Bioswales Design Review Check List

September 2015

Applicant: ____________________________________	Date: _________________________________

Submitted By: ________________________________	Project Location: _______________________

- Drainage Area _____________SF and  __________________Ac

- How much of the the DA is Impervious Surface _______________% and  _____________SF (if soil quality restoration is done or if soils investigations indicate green space is capable of absorbing the WQv the green space can be eliminated from the DA for WQv calculation. If neither applies, assume ½ of the green space is equivalent to impervious surface.)

- Discuss soils investigation findings (i.e. texture, degree of compaction, percolation potentials, depth to water table, etc) ______________________________________________________________________ ____________________________________________________________________________________

____________________________________________________________________________________WQv __________________CF (show calculations below or attach a copy)                                                   WQv= (Rv) x (P)x (DA) x 43,560 SF/ac x (1 ft/12in)		(See Iowa SW Mgt Manual)

- Type of cross section (check one) _____ Trapezoidal _____ Parabolic

- Length of Bioswale __________________Ft  (show calculations below or attach a copy)

- Lbioswale = Tres x V x (60 sec/min)				(See Iowa SW Mgt Manual)

- Bottom Width (if trapezoidal)_____________________Ft

- Side Slopes (if trapezoidal)_____:______

- Top width (if parabolic) ____________ Ft

- Grade of Swale_____________%

- Residence time _____________minutes

- Velocity _______________fps

- Type of berm (rock or earth) _____________________________

- Height of berms __________________________inches

- Spacing of berms (from toe of upper berm to ridge of the next lower berm) ________ ft.

- Describe the soil media. (Soil blend specified in the Iowa Stormwater Management Manual is 75% - 90% washed concrete sand, 0-25% topsoil, 0-10% organic material):

  - Sand 			__________%

  - Topsoil 		__________%

  - Organic material 	__________%

- Quantities (please attach a copy of materials calculations)

  - Sand 			_____________tons;

  - Topsoil			_____________tons or CY;

  - Organic material	_____________tons or CY

- Depth of Rock Chamber the tile is bedded in ________________inches. Quantity & Type of Rock ____________tons of ________________________________

- Was the tile trench filled with soil media or choker material (3/8” chip)

- Quantity and Type of choker material _______tons of ___________________________

- Size of perforated drain tile _____________inch

- Describe any  pretreatment techniques provided (what practice(s) were used, how were things sized, etc) _______________________________________________________________________________ ____________________________________________________________________________________

- Describe outlet for the bioswale and the perforated drain tile __________________________________ ____________________________________________________________________________________

____________________________________________________________________________________

- Spacing of plants ___________

- Size of plants ________________

- Quantity of plants __________ (Please attach a plant list and planting plan)

- If seeding was done describe type and quantity of seed used and the rate that was applied  (i.e lbs/ac or per 1,000 SF _______________________________________________________________________

____________________________________________________________________________________

- Describe the erosion control installed to protect the bioswale until vegetation is established _________

________________________________________________________________________________________________________________________________________________________________________

- If residence time and velocities that manage the water quality volume (WQv) can’t be achieved, describe the treatment train components that will be added to manage the WQv ____________________________________________________________________________________

____________________________________________________________________________________

- Please attach a map of the drainage area.

- Please attach a plan view, profile and cross sectional drawing

FOR REVIEWERS USE ONLY

This design appears to comply with the standards in the Iowa Stormwater Management Manual.

This design does not appear to comply with the standards in the Iowa Stormwater Management Manual.

Comments: ____________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

Name of Reviewer: __________________________________________	Date: _____________________

Signature: ____________________________________________________________________________