Soil Management / Soil Quality Restoration for New Lawns (SQR)

September 15, 2015

Applicant______________________________	Date________________________________

Submitted by___________________________	Location_____________________________

- What method from the Iowa Stormwater Management Manual’s (ISWMM) chapter on soil management and soil quality restoration will be / was used to treat the area (Method 1-7) _____________. (Google “iowa stormwater management manual” and it will take you to the DNR website that houses the manual.)

- What is the water quality volume (WQv) that will need to be managed? (show calculations below)  _____________gallons and ______________CF.

________L x _________W of drainage area / 43,560 SF per ac = ____________ac of DA

________Ac x 27,152 gals/ac/in of rain = _______________ gallons/ac/in

________Gallons/ac/in x 1.25 inches = _________________ WQv in gallons

________Gallons/WQv x 0.1337 CF/gal =________________ WQv in CF

- Yes or No.  Does the area being treated with SQR only manage rain that falls directly on If the area it? (If yes – proceed to question 7. If no – proceed to question 4).

- If the area being treated will manage runoff from impervious surfaces show calculations for determining that the storage capacity of the treated area meets or exceeds that needed for the WQv (1.25 inches of rain for the lawn and the area of impervious surface draining to it…see example below). Or, provide documentation of the organic matter (OM) content and depth of decompacted soil profile and the capacity of the soil to absorb the WQv from Table 6 in the SQR chapter of the ISWMM. (See example below.)

Example: Assume Method 7 will be used (2 inches of compost is incorporated to a depth of 8”). Assume compost will have an OM of 40%.

Then – 2” compost / 8” of profile = 25% of the profile @ 40% OM = 10% OM content.

Go to table 6 in the soil management/SQR chapter – see the % Soil OM column – use 8% OM – go to the far right column (8” depth) and see that there is 3.1 inches of available water storage.

Subtratct the WQv from the available water storage (3.1-1.25 = 1.85 inches of available storage above the WQv.

- From Table 8 of the SQR chapter show the excess water storage volume ________inches and the factor to determine the maximum impervious surface that can be treated _________. Go to table 8 and see that the excess water storage will allow 1.26 to 1.68 times the available green space that can shed the WQv onto the treated area – or – for every 100 SF of treated area anywhere from 126-168 SF of adjacent impervious surface could shed the WQv and be absorbed. You could pin this down to a more precise factor using the following formula:

- 1.85 inches of available excess storage represents 92.5% of the excess water storage available in the range between 1.5 to 2.0 inches of potential excess storage..

- The factor from Table 8 to determine is 1.26 to 1.68 or a difference of 0.42. So you could use 92.5% of 0.42 (or 0.38) and add it to the 1.26 for a factor of 1.42…or for every 100 sq ft of green space you can shed the WQv from 142 sq ft of impervious surface

- How many square feet of impervious surface can / will be managed by the soil management / SQR area _________________________________.

- Provide the quantities of materials applied as amendments

  - Sand 		_______ tons

  - Topsoil 	_______ tons / CY

  - Compost	_______ tons / CY

- Provide a copy of the planting plan with quantities of seed or plants used and a listing of species and rates applied._________________________________________________________________________

________________________________________________________________________________________________________________________________________________________________________

- Describe the erosion and sediment control measures used to protect the site until vegetation is established ________________________________________________________________________________________________________________________________________________________________________

- Please attach a map or aerial photo of the soil management / SQR area.

FOR REVIEWERS USE ONLY

Design appears to comply with applicable design standards, and local, state, and federal requirements.

Design does not appear to comply with applicable design standards, and local, state, and federal requirements.

Comments:______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Name of Reviewer: __________________________________________ Date:_____________________

Signature: ____________________________________________________________________________